Appendix 2

| Appendix 2. ERG protocol settings. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ERG response | Cell | Light source | Light intensity Log Cd ms-2 | Stimulus lenght msec | Basal time recording msec | After stimulus time recording | High pass filter Hz | Low pass filter Hz | average |  |
| DSR | GCL | Ganzfeld | -4 | 5 (single) | 100 | 500 | 1 | 1000 | 10 |  |
| b-scotopic | Rod | Ganzfeld | -2 | 5 (single) | 100 | 500 | 1 | 1000 | 10 |  |
| Mixed | Rod+cones | Diode | 1.5 | 5 (single) | 100 | 500 | 1 | 1000 | 10 |  |
| Oscilatory potentials | Inner retina neurons | Diode | 1.5 | 5 (single) | 100 | 500 | 30 | 10000 | 10 |  |
| b-photopic | Cones | Diode | 2 | 5 (single) | 100 | 500 | 1 | 1000 | 6 |  |
| Flicker | Cones | Diode | 2 | 5 (20Hz) | 100 | 500 | 1 | 1000 | 6 |  |